# PHOENIX Automatic Return Drop Test System Plan

Operations and software plan for a 1.5 kg inert recovery article using a steerable brake line parafoil

Planning baseline

Revision 10 October 2026

Flare disabled

This plan defines the intended automatic return drop test and the evidence required before steering is enabled. The current system is not ready for a guided aerial test. A purpose built ram air parafoil rated for the complete suspended mass must replace the ordinary round parachute, and every brake limit must be measured before use.

This document does not authorize a building drop or powered rocket flight. A softened landing surface does not control the risks created by release equipment, wind drift, spectators, structures, roads, or power lines. Any elevated release site requires a qualified adult test lead, property authorization, a reviewed release system, and a calculated exclusion area.

Primary development rule  Prefer a stable landing away from Home over an unstable attempt to land exactly on Home.

# Document contents

1  Decision baseline

2  System architecture

3  Operational state machine

4  Guidance and brake control

5  Sensor roles and fault behavior

6  Dashboard and communications

7  Data recording and evidence

8  Progressive test phases

9  Acceptance and unlock criteria

10  Current implementation gaps

11  Experimental calibration register

12  Operating checklists

13  Evidence sources and limitations

# How to use this plan

The test team should treat each phase as a gate. A later phase remains locked until software checks pass and a human reviewer confirms the log, synchronized video, hardware inspection, and site conditions. A successful landing does not compensate for missing data, an unexplained reset, a wrong brake mapping, or an unstable canopy.

# 1 Decision baseline

The following decisions consolidate the completed planning questions. Values that depend on the real canopy remain unset until measured. Where a requested behavior could create an uncontrolled condition, this plan adds a hard neutral or stability guard without changing the intended automatic return objective.

| Area | Agreed design |
| --- | --- |
| Test article | Complete 1.5 kg inert recovery article. No motor, ignition, or energetic deployment device is part of the drop test. |
| Canopy assumption | Purpose built steerable ram air parafoil with separate left and right brake lines, rated by its manufacturer for the complete suspended mass. |
| First release configuration | Canopy is open and inflated before release. The crew visually confirms readiness and records that confirmation on the dashboard. |
| Target | The dashboard captures the current stable GPS position as Home. Several plausible fixes are required before Home is accepted. |
| Release geometry | The system may be released from different directions. Return to Home is not guaranteed when altitude, wind, heading, or glide performance makes the target unreachable. |
| Arming | A physical payload arming action is followed by two dashboard confirmations. Unhealthy required sensors allow only a neutral brake recording test. |
| Emergency command | A received Abort to Neutral command latches both brakes at neutral. Later guidance code cannot overwrite it. |
| Brake scale | Zero percent is calibrated neutral. One hundred percent is the experimentally measured safe brake travel, never raw servo endpoint travel. |
| Test progression | Neutral, then 20 percent, then 40 percent, then the validated full safe range. Each level requires objective checks and human review. |
| Final approach | Steering authority decreases, heading deadband increases, and flare remains disabled. |
| Logging | The payload keeps the authoritative full rate log. LoRa sends a lower rate preview. After landing, Wi Fi or USB retrieves the completed log. |
| Reboot | A brownout or reset commands immediate neutral, records the reset reason, enters fail neutral descent, and continues recovery telemetry when possible. |

## Nonnegotiable constraints

The existing LOC Precision 24 inch or 28 inch round parachute does not satisfy the steerable parafoil assumption merely because suspension lines can be pulled.

The BMP388 barometer cannot identify left and right. It reports pressure and temperature, from which altitude and vertical speed are estimated.

The BNO085 supplies motion and orientation information. Its axes must be aligned to a saved payload reference before they are used for release, stability, or course hold logic.

IMU only operation cannot navigate to a geographic coordinate. Without GPS position, the controller can only hold the last valid course within a restricted authority envelope.

No drop height in this plan is an approval. The previously discussed 15 to 30 m range is only a proposed test envelope and may be unsuitable after canopy, site, and trajectory review.

# 2 System architecture

The payload owns timing, sensing, state transitions, servo commands, and the authoritative log. The ground unit hosts the operator dashboard and exchanges commands and preview telemetry over LoRa. Autonomous control must not depend on Wi Fi, LoRa, or a connected computer after arming.

| Component | Responsibility | Failure behavior |
| --- | --- | --- |
| Payload Heltec WiFi LoRa 32 V4.3 | Runs the coordinator, sensor processing, guidance, servo output, local log, and post landing recovery services. | Continues autonomously after ground link loss. Reboot starts neutral. |
| Ground Heltec WiFi LoRa 32 V4.3 | Creates the PHOENIX GROUND Wi Fi network, serves the dashboard, sends discrete commands, and displays preview telemetry. | Its loss cannot change the payload control state. |
| BNO085 | Release candidate, orientation reference, angular rate, oscillation detection, and restricted last course hold after GPS loss. | Invalid or stale data forces neutral when guidance is active. |
| BMP388 | Pressure, filtered altitude AGL, vertical speed, release confirmation, stability evidence, final approach altitude, and landing evidence. | Invalid or stale data blocks release confirmation or forces neutral during active guidance. |
| L76K GNSS | Position, ground speed, course over ground, satellite and fix quality, Home capture, target bearing, distance, and reachability evidence. | Guidance cannot navigate to Home. Restricted IMU course hold may continue under the GPS loss rules. |
| Servo 1 and Servo 2 | Pull the two brake lines through calibrated pulse and line travel limits. Names do not assume physical left or right. | Any invalid command, control fault, or latched abort commands calibrated neutral. |

## Communications behavior

The ground Wi Fi access point starts automatically with the ground controller and serves the local dashboard.

LoRa carries low rate telemetry and discrete commands. Missing packets never freeze a previous steering command at the payload.

During descent, the only ground command with control priority is the authenticated, latching Abort to Neutral command.

The payload Wi Fi service is reserved for post landing log retrieval under this plan. It must not become a flight dependency.

USB remains the recovery path for firmware service and log retrieval if wireless download is unavailable.

# 3 Operational state machine

The automatic return profile is separate from the powered launch profile and from the neutral drop recorder. The launch profile remains locked until inert testing demonstrates repeatable deployment, stable neutral descent, conservative steering, deterministic neutral behavior, and reliable logging.

| State | Required evidence | Servo behavior | Exit |
| --- | --- | --- | --- |
| Safe idle | Power on, reset reason captured, calibration loaded and validated. | Neutral | Physical arm action |
| Preflight | Sensor health, servo rail health when measurable, valid configuration, adequate log space. | Neutral | Dashboard confirmations |
| Armed buffering | Home accepted for guided mode; visual canopy confirmation recorded; circular buffer running. | Neutral | IMU release candidate |
| Release candidate | IMU detects release motion. The event is timestamped immediately. | Neutral | Barometric descent confirms within the configured window |
| Release confirmed | Sustained filtered descent and altitude loss. A single pressure spike cannot satisfy the guard. | Neutral | Settling interval expires |
| Stabilization | Established descent, acceptable angular rate, limited vertical speed variation, valid inputs. | Neutral | Stable window persists |
| Guided descent | Valid target, GPS position and course, sufficient ground speed, reachability estimate, healthy IMU and barometer. | Slew limited differential brake | Final approach, landing, abort, or fault |
| GPS course hold | GPS invalid after guidance began; IMU remains healthy; last GPS course is plausible. | Restricted authority heading hold | GPS recovery, landing, instability, abort, or fault |
| Final approach | Altitude or distance threshold reached. | Reduced authority and larger deadband | Landing, abort, or fault |
| Landed | Five seconds of low vertical speed, small altitude span, low ground speed, and low IMU motion. | Neutral | Finalize log and enable recovery services |
| Fail neutral | Critical sensor, servo, control, reset, data quality, or safety guard failure. | Neutral latched | Remain until landing or deliberate ground reset |

## Release and canopy evidence

The IMU provides the fast release candidate and the barometer confirms sustained downward motion. This ordering reduces trigger delay without allowing one acceleration transient to start guidance. For the first campaign, canopy readiness is a crew observation made before release. Sensor signatures may record loading or settling, but they do not prove that a canopy is fully open.

# 4 Guidance and brake control

## Navigation calculation

When GNSS ground speed is high enough for a trustworthy course, the controller uses course over ground as its navigation heading. It recalculates target bearing and distance on each accepted GPS update. Heading error is the normalized difference between target bearing and current course, limited to minus 180 through plus 180 degrees.

A negative heading error requests the brake mapped to a left turn.

A positive heading error requests the brake mapped to a right turn.

An error inside the active deadband requests neutral.

Proportional gain, command smoothing, slew rate, reversal delay, saturation, and stability scaling are applied before a servo pulse is produced.

## Servo direction and orientation setup

The dashboard keeps the actuator names Servo 1 and Servo 2. During setup, the payload rests in its installed orientation and the operator presses Set Current Orientation as Zero. The operator then traces the two brake lines and assigns each servo to the observed turn direction. Small controlled movements and later IMU or GPS response verify the saved mapping. The barometer is not part of left right identification.

## Brake authority

The software percentage is a calibrated line travel scale. The operator first measures neutral pulse width and neutral line position, then establishes a mechanical limit without canopy load, and finally derives a lower canopy safe limit from controlled tests. The software maps 100 percent to that canopy safe limit. No control mode may send a pulse outside the saved minimum and maximum values.

| Level | Purpose | Unlock evidence |
| --- | --- | --- |
| Neutral | Measure natural inflation, glide, sink, oscillation, release timing, landing detection, and logging. | Repeatable stable descents and complete records |
| 20 percent | Confirm turn direction, delay, gentle response, neutral return, and left right asymmetry. | Correct direction with no instability or prolonged saturation |
| 40 percent | Measure stronger response, turn rate, altitude loss, reversal behavior, and stability margin. | Stable response and conservative margin below abnormal behavior |
| Full safe range | Run automatic return using the highest travel demonstrated safe for this exact canopy and mass. | Software checks plus human review of repeated 40 percent evidence |

## GPS loss behavior

The selected plan retains a fixed maximum authority for IMU based last course hold until GPS returns. This does not mean a fixed nonzero brake pulse. The heading controller varies the brake command to hold the last valid course and returns toward neutral whenever the course error is small. It cannot update distance to Home or correct for wind drift while GPS position is unavailable.

The mode terminates at landing, on IMU invalidity, excessive angular motion, repeated oscillation, implausible elapsed course hold, an abort command, or any critical fault. Replay and hardware in the loop testing must establish the authority limit before this behavior is enabled. Until then, the implemented behavior should remain fail neutral.

## Unreachable target behavior

If Home appears unreachable, the controller may attempt the strongest calibrated safe turn requested in the planning decisions. The attempt ends when the payload aligns toward Home, enters final approach, reaches a low altitude guard, exceeds the angular motion or oscillation limit, loses required sensor validity, or triggers a reversal guard. If the target remains unreachable after the guarded attempt, stable descent takes priority over continued maximum braking.

## Final approach and flare

Final approach begins at experimentally derived altitude or distance thresholds. The controller reduces steering authority, increases the heading deadband, avoids rapid reversals, and stops chasing small GPS errors. Automatic flare remains disabled in every drop test and launch profile covered by this plan.

# 5 Sensor roles and fault behavior

| Condition | Controller response | Recorded evidence |
| --- | --- | --- |
| Required sensor unhealthy before arming | Allow only neutral brake recording mode. | Validity, age, fault code, operator decision |
| GPS unavailable before guidance | Do not start geographic guidance. Remain neutral. | Fix state, satellites, HDOP, NMEA age |
| GPS lost after guidance | Enter restricted last course hold under the GPS loss rules. Do not claim continued target navigation. | Last accepted position and course, loss time, hold commands |
| GPS jump | Reject the point, retain the last accepted position briefly, reduce authority, and require several plausible fixes. | Rejected position, jump distance, recovery count |
| IMU invalid or stale | Neutral latch during active guidance. IMU course hold is unavailable. | Sensor age, quaternion status, fault code |
| Excessive rotation or oscillation | Reduce authority while continuing guidance. If the absolute guard persists or worsens, neutral latch. | Angular rate, sign reversals, authority scale |
| Barometer invalid or stale | Do not confirm release or use altitude state changes. During active guidance, neutral latch unless a separately validated degraded mode exists. | Pressure, altitude age, rejection reason |
| Barometer spike | Reject the sample. Do not change state from one sample. | Raw value, filtered value, rejection marker |
| Servo command saturation | Clamp to saved safe travel, log saturation, and avoid integral or repeated command growth. | Requested and actual command, pulse, duration |
| Wi Fi or LoRa loss | No change to autonomous sensing or guidance. | Link status, packet counters, RSSI and SNR when available |
| Abort received | Latch neutral immediately. No later loop can overwrite it. | Command sequence, authentication result, latch timestamp |
| Brownout or reset | Neutral immediately, record reset reason, fail neutral descent, recovery telemetry when possible. | Reset cause, configuration version, boot time |
| NaN or impossible value | Reject it, mark source unhealthy, never pass it to navigation or control. | Value source, reason, age of last good value |

## Sensor reference rules

Zero the BNO085 only after the payload is secured in its flight orientation. Save the mounting convention with the configuration version.

Treat BMP388 altitude as a filtered relative estimate. Pressure disturbances, temperature, mounting, and airflow can change it.

Accept Home only after several stable GPS fixes. Required satellite count, accuracy, HDOP, and persistence remain calibration parameters.

Use GPS course only above a measured minimum ground speed. At lower speed, increase deadband and avoid aggressive corrections.

Every sample used by the coordinator must carry a validity flag, payload timestamp, and age.

# 6 Dashboard and communications

The operator connects to PHOENIX GROUND for live supervision. The dashboard sends discrete LoRa commands to the payload and displays payload timestamps. It does not close the flight control loop. Dashboard closure, phone sleep, Wi Fi disconnection, and ground power loss cannot stop payload sensing or overwrite a payload state.

| Dashboard area | Required functions |
| --- | --- |
| System status | Payload link, configuration version, reset reason, battery and servo rail voltage when hardware exists, sensor validity and age, log capacity, and current state. |
| Home | Capture current position, show stability count and fix quality, reject invalid coordinates, display bearing and distance, and freeze Home after arming. |
| Orientation and servos | Set current BNO085 orientation as zero, display axes, show Servo 1 and Servo 2 calibration, assign turn direction, test neutral and small steps, and save mapping. |
| Canopy readiness | Record crew visual confirmation that the pre opened canopy is laid out, inflated, untangled, and clear of the release structure. |
| Arming | Require physical payload arm state, preflight checklist, first confirmation, five second confirmation window, selected authority level, and guided or neutral profile. |
| Active test | State, timers, sensor health, Home geometry, requested and actual brake commands, angular motion, altitude AGL, vertical speed, GPS course, reachability, LoRa health, and prominent abort control. |
| Evidence | Test ID, local log status, event timeline, payload file checksum, video filename, review result, anomaly list, and next level unlock state. |
| Recovery | Final coordinates, recovery beacon status, payload Wi Fi download status, USB fallback instructions, and log export controls. |

## Command rules

Every safety relevant command carries a sequence number and acknowledgement.

Arming and authority changes require confirmation. Home, calibration, and profile changes are rejected after arming.

Manual steering and bench servo commands are disabled while a drop recording is active.

Abort to Neutral remains available during the test and latches at the payload after one valid receipt.

A missing acknowledgement is shown as unknown delivery, not as success.

# 7 Data recording and evidence

Payload time is authoritative. The payload maintains a circular pre release buffer, commits it when release is detected, records the entire descent, keeps a post landing interval, and closes the file only after the final event record is written. The pre release duration remains configurable in the 30 to 60 second range until the flash budget and write endurance are verified.

| Data group | Target rate | Minimum fields |
| --- | --- | --- |
| IMU | 50 Hz | Payload timestamp, validity and age, roll, pitch, yaw, quaternion status, gyro X Y Z, angular rate, acceleration X Y Z |
| Servo | 50 Hz | Requested and actual Servo 1 and Servo 2 commands, pulse widths, slew state, saturation, reversal guard, authority scale |
| Barometer | 25 Hz | Pressure, temperature, raw and filtered relative altitude, vertical speed, validity, age, rejection state |
| GNSS | Verified native rate | Latitude, longitude, altitude, fix type, satellites, HDOP or accuracy, ground speed, course, validity and age |
| Coordinator | Every state change and periodic snapshot | State, mode, test ID, event code, fault code, target validity, reachability, target bearing, heading error, distance |
| Power | As hardware supports | Battery voltage, servo rail voltage, validity flags, minimum observed voltage, reset cause |
| Radio | Preview rate and packet event | Sequence, command acknowledgement, RSSI, SNR, packet loss and link age |

## Test deliverables

A human readable CSV with units and documented columns.

A raw replay file that preserves all values needed to run the actual coordinator deterministically.

An event report containing state transitions, rejections, faults, reset causes, saturation, and command acknowledgements.

A configuration snapshot containing firmware commit, configuration version and checksum, calibration source, canopy identification, mass, and selected authority level.

Summary charts for altitude, vertical speed, course and target bearing, heading error, angular rate, brake commands, sensor validity, and power when measured.

A whole file checksum and an explicit indication that the log closed successfully.

# 8 Progressive test phases

Each phase uses the same coordinator and logging schema intended for later tests. The test article remains inert. Skipping a phase is not allowed because later authority levels depend on measurements produced by earlier phases.

## Phase 0 Canopy and site qualification

Obtain manufacturer documentation for a steerable ram air canopy rated for the 1.5 kg suspended mass. Record canopy model, area, line plan, risers, brake travel guidance, packing or pre inflation instructions, and limitations. A qualified adult test lead reviews the release site and trajectory area. An ordinary building remains no go until the release structure, roof or platform access, exclusion area, wind limits, and recovery area are formally reviewed.

## Phase 1 Host replay and software integration

Run the real coordinator using synthetic and recorded inputs. Verify release confirmation, stabilization, guidance gates, target validity, heading wraparound, course hold, reachability, final approach, landing, abort, sensor failures, GPS jumps, resets, and log closure. A failsafe or abort command must not be overwritten later in the loop.

## Phase 2 Powered bench calibration

With the article restrained and canopy brake load disconnected, measure neutral pulse, mechanical travel, line displacement, reversal, current draw, servo rail voltage, and response delay. Run sensors, LoRa, logging, and servos together for longer than the expected test. Save calibration and verify it after reboot.

## Phase 3 Suspended static and tethered testing

Install the representative harness and mass. Confirm that the structure carries the payload and servos pull only the brake lines. Verify line routing, neutral geometry, small command response, abort, loss of links, sensor unplug faults, and no brownout. Do not infer aerodynamic turn performance from a static pull.

## Phase 4 Pre opened neutral drops

Release with the canopy already open and visually confirmed. Both servos remain neutral. Measure natural forward speed, sink rate, glide ratio, oscillation, heading stability, release detection delay, stable descent timing, landing detection, and log integrity. Use synchronized video and post test inspection.

## Phase 5 Twenty percent guided drops

Enable a maximum of 20 percent of the saved canopy safe travel. Confirm Servo 1 and Servo 2 mapping, response delay, direction, neutral return, left right asymmetry, altitude loss in turns, reversal behavior, and fault neutral response. Begin with geometry that permits a gentle correction; later runs may vary direction.

## Phase 6 Forty percent guided drops

After Phase 5 passes, enable 40 percent. Measure turn rate, radius, sink increase, oscillation, saturation, and stability margin. Exercise guarded turn back, GPS loss, gust replay, and final approach scaling without enabling flare.

## Phase 7 Validated full safe automatic return

Map 100 percent to the highest experimentally demonstrated safe travel, not the servo endpoint. Test varied bearings, wind conditions within the approved envelope, unreachable target handling, GPS recovery, and conservative final approach. Evaluate landing error only after stability and fault behavior remain repeatable.

## Phase 8 Separate launch recovery qualification

Keep launch settings in a different profile. Demonstrate mechanical deployment and guidance separately before combining them. Powered ascent never steers. Launch guidance remains locked until independent review confirms representative inert deployment, full mass recovery, power margin, logging, exclusion zone, and club or site approval.

# 9 Acceptance and unlock criteria

The dashboard may recommend a pass, but a human reviewer makes the progression decision after comparing the payload log, video, hardware inspection, site record, and test objective. Reaching Home is not enough to pass.

| Category | Pass requirement |
| --- | --- |
| Configuration | Firmware commit, configuration checksum, calibration version, canopy identification, mass, profile, and authority level are recorded and unchanged during the run. |
| Release | No false release before the physical event. IMU candidate and barometric confirmation have plausible payload timestamps. |
| Canopy | Crew visual confirmation exists. Video and inspection show no line crossing, damaging inflation, collapse, or structural load carried by servos. |
| Sensors | Required data remain valid and fresh for the evaluated state. Rejected samples and dropouts are explained. |
| Control | Brake direction is correct; slew and reversal guards work; no pulse exceeds calibration; saturation is absent or brief and explained. |
| Stability | Angular motion and heading sign changes remain within the measured envelope. Any excessive motion causes the selected authority reduction and absolute neutral guard. |
| Safety behavior | Abort, injected faults, link loss, and reboot produce the expected neutral or restricted behavior. No later code overwrites a latch. |
| Power | No brownout, reset, overheating, rail collapse, or sensor corruption occurs under combined load. |
| Logging | Pre release data, events, descent data, landing confirmation, post landing interval, configuration, and checksum are present. Replay reproduces the decisions. |
| Landing | The article lands within the cleared area without an unsafe late correction. Accuracy is recorded but is not the primary early phase pass criterion. |
| Review | Video and log are reviewed, anomalies receive dispositions, and the qualified test lead authorizes either repeat or progression. |

## Automatic software checks

No command outside calibrated pulse and travel limits.

No nonzero guidance before release, settling, stabilization, and navigation gates complete.

No guidance with missing Home or implausible GPS position.

No prolonged saturation or prohibited rapid turn reversal.

No NaN or impossible value reaches bearing, distance, reachability, or servo calculations.

Landing remains confirmed for five seconds before the log closes.

Flare is false in the configuration snapshot and unreachable in replay.

# 10 Current implementation gaps

The repository already contains a neutral drop recorder, a calibration locked guided drop controller, LoRa commands, payload logging components, and a ground dashboard. The completed planning choices are not all implemented. The following differences must be resolved before the plan describes the running firmware.

| Gap | Current repository behavior | Required change |
| --- | --- | --- |
| Guided profile lock | GUIDED DROP TEST ENABLED and calibration confirmed are false. | Keep the lock until physical calibration and earlier test gates pass. |
| Authority | Guided drop is presently capped at 15 percent and final approach at 8 percent. | Implement staged 0, 20, 40, and validated full safe profiles with evidence based unlocks. |
| Activation altitude | Minimum guided activation is a placeholder 75 m. | Derive any lower threshold from measured stabilization time and site review. Do not lower it merely to fit a 15 to 30 m release. |
| GPS loss | The current guided controller slews toward neutral and latches fault neutral after about two seconds. | Design, replay, and validate the selected restricted IMU last course hold before replacing fail neutral behavior. |
| Direction mapping | Servo channels use existing calibration and reversal fields. | Add BNO085 orientation zero, explicit Servo 1 and Servo 2 turn mapping, persistence, and verification workflow. |
| Payload Wi Fi | The flight build currently disables payload Wi Fi and directs users to PHOENIX GROUND. | Add a post landing only payload download service or retain USB retrieval until that service is verified. |
| Log download | The command path can request a log index, but complete chunked transfer and checksum verification are not finished. | Implement resumable download, whole file checksum, CSV conversion, replay export, and clear completion status. |
| Logging rates | The desired 50 Hz, 25 Hz, and native GNSS recording schedule is a plan requirement, not verified behavior. | Measure flash throughput, file size, timing jitter, and data loss under servo and radio load. |
| Power measurements | Battery and servo rail fields are valid only when measurement hardware is fitted and calibrated. | Do not synthesize values. Add and calibrate sensing hardware if voltage evidence is required. |
| Canopy | The available LOC round parachute is not a steering qualified ram air canopy. | Obtain and document a suitable canopy before aerodynamic calibration or guided tests. |

## Implementation milestone

The immediate software milestone is complete when synthetic or replayed data drives the real coordinator through arming, release, stabilization, guidance, final approach, and landing; each staged profile produces the expected limited command; every injected critical fault neutralizes the servos; calibration survives reboot; and the complete decision history can be downloaded and replayed.

# 11 Experimental calibration register

Every item below remains marked Requires Experimental Calibration until the final hardware, canopy, line geometry, mass, power system, and firmware configuration produce repeatable measurements.

| Parameter | How it is measured | Minimum evidence before use |
| --- | --- | --- |
| Servo 1 and Servo 2 neutral pulse | Unloaded linkage and neutral brake geometry | Repeated measurement and reboot restoration |
| Mechanical pulse limits | Small pulse increments with load disconnected | No binding, stall, horn flex, or structural contact |
| Canopy safe brake travel | Suspended and progressive aerial tests in millimeters and pulse width | Conservative margin below any abnormal response |
| Turn mapping | Physical line trace, operator assignment, BNO085 zero, controlled response verification | Consistent left and right results |
| Minimum useful brake | Repeated low authority turns | At least three consistent responses per side |
| Response delay | Payload command timestamp to video, IMU, and GPS response | Distribution across both directions and relevant voltage |
| Turn rate and radius | Course and yaw change during steady commands | Repeated values with asymmetry recorded |
| Neutral sink rate | Filtered barometer and GPS altitude where credible | Multiple stable neutral descents |
| Turn sink penalty | Compare vertical speed at each authority level | Conservative value for reachability and altitude guards |
| Forward speed and glide ratio | Ground tracks in reciprocal conditions with wind estimate | Median and conservative bound |
| Stabilization limits | Angular rate and vertical speed variation after release | No guidance until stable window is repeatable |
| GPS course speed threshold | Compare course noise against ground speed | Threshold below which authority and trust are reduced |
| Final approach thresholds | Derived from delay, turn radius, sink rate, glide ratio, and target geometry | Validated at lower authority first |
| Landing thresholds | Stationary and real touchdown records | No false landing in descent and reliable five second confirmation |
| Wind and site envelope | Measured wind, trajectory reconstruction, and cleared area | Qualified test lead approval for each series |

# 12 Operating checklists

## Before bringing the article to the release point

□ Confirm the exact test phase and its authority lock.

□ Confirm the article is inert and contains no motor, ignition circuit, or energetic deployment device.

□ Record total suspended mass and configuration version.

□ Inspect harness, structural suspension, risers, brake lines, servo horns, battery restraint, wiring, and antenna connections.

□ Verify common ground and servo rail voltage under commanded movement.

□ Confirm Servo 1 and Servo 2 neutral, direction mapping, pulse limits, and saved calibration after reboot.

□ Obtain stable outdoor GNSS fixes and capture Home only after the acceptance rule passes.

□ Verify BNO085 zero reference and BMP388 relative altitude baseline.

□ Confirm payload flash capacity, test ID, payload clock, ground LoRa link, and dashboard operation.

□ Confirm weather, wind, exclusion area, recovery route, property authorization, test lead, and emergency stop roles.

## At the release point

□ Lay out and inspect the steerable parafoil according to its manufacturer documentation.

□ Confirm the canopy is open, inflated, untangled, and clear of the structure; record visual confirmation.

□ Confirm nobody and no property is inside the predicted path or exclusion area.

□ Perform the physical arm action, review the dashboard warnings, and complete both confirmations.

□ Verify Armed Buffering, neutral servo commands, active local log, frozen Home, and the selected authority level.

□ Release without holding or disturbing brake lines. Do not attempt live manual steering.

## After landing

□ Wait for Landed state, neutral confirmation, log closed indication, and recovery coordinates.

□ Approach only after the test lead confirms the area is safe.

□ Photograph the canopy, lines, harness, servos, battery, and structure before moving them when practical.

□ Download the raw replay file, CSV, event report, configuration snapshot, and charts. Verify the checksum.

□ Save synchronized video and record its filename in the event report.

□ Review resets, sensor gaps, saturation, angular motion, sink rate, turn response, neutral return, and landing detection.

□ Mark Pass, Repeat, or Stop. Do not unlock the next level until software checks and human review both pass.

# 13 Evidence sources and limitations

The sources below support the development method: phased testing, GPS based navigation, onboard recording, simulation, instrumented reconstruction, and calibration from measured response. They do not validate PHOENIX dimensions, drop height, brake travel, gains, or landing accuracy. NASA vehicles and loads differ greatly from this 1.5 kg article.

NASA Technical Memorandum 4599  Further Development and Flight Test of an Autonomous Precision Landing System Using a Parafoil

Phased autonomous parafoil testing using GPS navigation, a flight computer, yaw sensing, and onboard recording.

NASA  An Overview of the Guided Parafoil System Derived from X 38 Experience

Instrumentation, simulation validation, postflight reconstruction, autonomous steering, and extensive subscale and full scale testing.

NASA  Parachute Testing for the NASA X 38 Crew Return Vehicle

Progressive test beds, instrumentation, guidance and control integration, and simulation.

Bosch Sensortec BMP388 product information and datasheet

The BMP388 is an absolute pressure and temperature sensor used for altitude tracking. It is not an axis or heading sensor.

CEVA BNO085 and BNO086 product information

The BNO085 is a nine axis sensor system used for motion and orientation information.

## Completion definition

Planning is complete when reviewers accept this document. The system is complete only after a suitable canopy is documented, software gaps close, calibration and test phases pass, and a test lead approves the site. Until then, guidance stays locked and flare stays disabled.